# ESP32 Programming Stack – Structured Cheat Sheet (ESP-IDF, WROOM-32)

This document explains the ESP32 software and hardware stack in logical execution order, starting from power-on and moving upward to application-level networking. Each section explains purpose, internal working, and essential API usage.

## 1. Complete Stack Overview

Application (app_main)
↓
ESP-IDF Components and APIs
↓
Device Drivers (GPIO, ADC, UART, SPI, I2C, WiFi, etc.)
↓
FreeRTOS Kernel
↓
HAL / LL Layer
↓
Memory-Mapped Hardware Registers
↓
ESP32 Silicon

All high-level API calls eventually write to hardware registers. Understanding this hierarchy allows structured debugging and controlled baremetal-style development.

## 2. Boot Process

1. Power On
2. ROM Bootloader executes (internal to chip)
3. Second-stage bootloader loaded from Flash
4. Partition table parsed
5. Application image copied to RAM
6. app_main() executed

ESP32 does not use main(). Execution begins in app_main(). Boot configuration depends on flash mode and partition layout.

## 3. Memory Architecture

Flash Memory: Stores firmware and partitions
IRAM: Instruction RAM for critical code
DRAM: Data RAM for variables and stacks
RTC Memory: Retained during deep sleep
ROM: Contains bootloader and low-level functions

Stack overflow and heap allocation errors are common sources of crashes. FreeRTOS tasks use separate stacks allocated from DRAM.

## 4. FreeRTOS Kernel

ESP-IDF runs on FreeRTOS. Every application runs as tasks scheduled by the kernel.

Task Creation:
xTaskCreate(task_function, "TaskName", 2048, NULL, 5, NULL);

Task Structure:
void task_function(void *pvParameters)
{
    while(1)
    {
    }
}

Delay:
vTaskDelay(pdMS_TO_TICKS(1000));

Scheduler selects highest priority ready task. Delay yields CPU to other tasks.

## 5. GPIO Stack

Header: #include "driver/gpio.h"

Configuration:
gpio_config_t io_conf = {
    .pin_bit_mask = (1ULL << 2),
    .mode = GPIO_MODE_OUTPUT,
    .pull_up_en = 0,
    .pull_down_en = 0,
    .intr_type = GPIO_INTR_DISABLE
};
gpio_config(&io_conf);

Set Output:
gpio_set_level(GPIO_NUM_2, 1);
gpio_set_level(GPIO_NUM_2, 0);

Driver configures IO_MUX and GPIO matrix internally. Direct register method:
GPIO.out_w1ts = (1 << 2);
GPIO.out_w1tc = (1 << 2);

## 6. ADC (SAR ADC)

Header: #include "driver/adc.h"

Configuration:
adc1_config_width(ADC_WIDTH_BIT_12);
adc1_config_channel_atten(ADC1_CHANNEL_6, ADC_ATTEN_DB_11);

Read:
int raw = adc1_get_raw(ADC1_CHANNEL_6);

12-bit ADC → Range 0–4095
Voltage = (raw / 4095.0) × 3.3V
LM35: Temperature (°C) = Voltage × 100

## 7. UART Stack

Header: #include "driver/uart.h"

Configuration involves:
- Baud rate
- Data bits
- Stop bits
- Parity

UART is used for serial debugging and communication.

## 8. I2C Stack

Header: #include "driver/i2c.h"

I2C uses SDA and SCL lines.
Supports master and slave modes.
Requires pull-up resistors.

## 9. SPI Stack

Header: #include "driver/spi_master.h"

SPI uses MOSI, MISO, SCLK, CS.
Full-duplex communication.
Common for displays and external ADC/DAC.

## 10. WiFi Stack

Headers:
#include "esp_wifi.h"
#include "esp_event.h"
#include "nvs_flash.h"

Initialization Flow:
nvs_flash_init();
esp_netif_init();
esp_event_loop_create_default();
esp_wifi_init(&cfg);
esp_wifi_set_mode(WIFI_MODE_STA);
esp_wifi_start();

WiFi driver controls radio.
TCP/IP handled by lwIP stack.

## 11. TCP/IP and Socket Layer

Header: #include "lwip/sockets.h"

Socket Creation:
int sock = socket(AF_INET, SOCK_STREAM, 0);
connect(sock, ...);
send(sock, "Hello", 5, 0);

TCP = reliable communication
UDP = connectionless communication

## 12. NVS (Non-Volatile Storage)

Header: #include "nvs_flash.h"

Used to store WiFi credentials and configuration parameters in flash.
Requires initialization before WiFi start.

## 13. Power Management

ESP32 supports light sleep and deep sleep.
RTC memory retains data in deep sleep.
Wake-up sources: timer, GPIO, touch sensor.

## 14. Build and Toolchain

idf.py set-target esp32
idf.py menuconfig
idf.py build
idf.py flash
idf.py monitor

Compiler: xtensa-esp32-elf-gcc
Build system: CMake + Ninja
Configuration stored in sdkconfig.